                                                DJI Mavic Air  Operator’s Checklist Ver 1.00                                                                                                             DJI Mavic Air Operator’s Checklist Ver 1.00


[image: image1.jpg]



MAVIC AIR
OPERATOR’S CHECKLIST

[image: image2.jpg]





PRE MISSION PLANNING
1. Firmware – Check up to date.
2. DJI GO 4 app - Check up to date.
3. Maps – Download as required.
4. Batteries fully charged – Check
5. Transmitter – Check fully charged.
6. Tablet / Phone / Device charged – Check for charge level appropriate for the flight.

7. SD Card – Check free space

8. All required cables – Check

9. Accessories – Check as required
10. Weather conditions – Check for appropriate local weather requirements.  

11. Airspace – Check to ensure planned flight is in accordance with FAA / ICAO airspace rules and regulations.

MAVIC AIR PRE FLIGHT

1. Battery – Check to ensure four green lights.

2. Mavic Air – Inspect for damage and check general condition and security.

3. Mavic Air – Unfold front arms, then rear arms.

4. Propellers – Install, tighten, and check locked.

5. Gimbal cover – Remove.
6. Gimbal security clamp - Remove
7. Transmitter – Unfold antennas and position upwards.

8. DJI GO 4 app – On and connected, login as required.
9. Transmitter – Connect to device as required.

10. Transmitter – P / S mode as required.

MAVIC AIR PRE LAUNCH

1. Transmitter – Turn on.

2. Mavic Air – Turn on.

3. DJI GO 4 app – Check connected.

4. Mavic Air / Transmitter – Check firmware up to date.

5. SD Card – Check free space, format if required.

6. Compass – Check, calibrate if required.

7. Gimbal – Check, calibrate if required.

8. Return to Home (RTH) height – Set as required, if indoors set RTH off.


9. Satellite Status – Check

10. Overall System Status – Check for any warning messages.

11. Launch Position – Check, ensure area is free from debris.

12. Home Point – Check, set.  Ensure the area directly above is free from obstacles.

MAVIC AIR LAUNCH
1. Take off area – Check clear.

2. Observers / Bystanders – Inform take off is imminent.

3. Battery power – Monitor.

4. Signal Strength – Monitor.

5. Motors – On.  Move both control stick down and in, or down and out.

6. Takeoff – Engage Auto Takeoff, or move left control stick up.

7. Hover check – Complete.  Hover for a period of 20-30 seconds.  Look and listen for any flight abnormalities.  

8. Home Point – Check, update as required.

MAVIC AIR LANDING

1. Landing area – Check clear, announce to observers / bystanders intent to land.

2. Return to Home (RTH) – activate if required or desired.  If not used, land aircraft manually.

AFTER LANDING

1. Mavic Air – Power down.

2. Gimbal security clamp – Install.

3. Gimbal cover – Install.

4. Transmitter – Power down, store.

5.  Mavic Air – Remove battery, propellers, and fold arms for storage.  Inspect aircraft for damage, and clean if required.


This Page Intentionally Left Blank
AIRCRAFT STATUS INDICATOR LIGHTS
NORMAL

R – G – Y
Alternate red, green, and yellow flashes.


Turning on, self-diagnostic testing.

Y – G 

Alternate yellow and green flashing.


Warming up.

G – SLOW 
Slow green flashing.


P-mode with GPS

G X 2

Two green flashes.
P-mode with forward and downward vision position.

Y – SLOW
Slow yellow flashing.
No GPS and forward and downward vision systems.

G – FAST
Fast green flashing.


Braking.


AIRCRAFT STATUS INDICATOR LIGHTS

WARNING
Y – FAST
Fast yellow light.


Remote control signal lost.

R – SLOW 
Slow red flashing.


Low battery warning.

R – FAST
Fast red flashing.


Critical low battery warning.

R – FLASH
Red flashing.


IMU Error.
R – SOLID
Solid red.


Critical Error.

R – Y

Fast alternating red and yellow flashing.


Compass calibration required.

WARNING


This operator’s checklist is intended for general reference only.  This is not an official DJI checklist.  Some conditions and/or situations may not be covered in these procedures.  By using this checklist, the user/operator acknowledges this warning and accepts full responsibility.








Questions or comments? Email the creator at �HYPERLINK "mailto:droneandhelopilot@gmail.com"�droneandhelopilot@gmail.com�





CAUTION


Launching the Mavic Air without a fully charged battery can result in undesired performance and intermittent power interruptions during return to home low battery flight.





WARNING


Operating in sport mode disables the collision avoidance system.  Be aware of surroundings.  Check for objects and people.





CAUTION


Ensure RTH height is set above surrounding obstacles.  









