WARNING

This operator’s checklist is intended for general reference only.  This is not an official DJI checklist.  Some conditions and/or situations may not be covered in these procedures.  By using this checklist, the user/operator acknowledges this warning and accepts full responsibility.

MAVIC AIR

Questions or comments? Email the creator at droneandhelopilot@gmail.com

OPERATOR’S CHECKLIST

PRE MISSION PLANNING

- Firmware – Check up to date.

- DJI GO 4 app - Check up to date.

CAUTION

Launching the Mavic Air without a fully charged battery can result in undesired performance and intermittent power interruptions during return to home low battery flight.

Maps – Download as required.

- Batteries fully charged – Check

- Transmitter – Check fully charged.

- Tablet / Phone / Device charged – Check for charge level appropriate for the flight.

- SD Card – Check free space

- All required cables – Check

- Accessories – Check as required

- Weather conditions – Check for appropriate local weather requirements.

- Airspace – Check to ensure planned flight is in accordance with FAA / ICAO airspace rules and regulations.

MAVIC AIR PRE FLIGHT

- Battery – Check to ensure four green lights.

- Mavic Air – Inspect for damage and check general condition and security.

- Mavic Air – Unfold front arms, then rear arms.

- Propellers – Install, tighten, and check locked.

- Gimbal cover – Remove.

- Gimbal security clamp - Remove

- Transmitter – Unfold antennas and position upwards.

- DJI GO 4 app – On and connected, login as required.

- Transmitter – Connect to device as required.

WARNING

Operating in sport mode disables the collision avoidance system.  Be aware of surroundings.  Check for objects and people.

Transmitter – P / S mode as required.

MAVIC AIR PRE LAUNCH

- Transmitter – Turn on.

- Mavic Air – Turn on.

- DJI GO 4 app – Check connected.

- Mavic Air / Transmitter – Check firmware up to date.

- SD Card – Check free space, format if required.

- Compass – Check, calibrate if required.

- Gimbal – Check, calibrate if required.

- Return to Home (RTH) height – Set as required, if indoors set RTH off.

CAUTION

Ensure RTH height is set above surrounding obstacles.

- Satellite Status – Check

- Overall System Status – Check for any warning messages.

- Launch Position – Check, ensure area is free from debris.

- Home Point – Check, set.  Ensure the area directly above is free from obstacles.

MAVIC AIR LAUNCH

- Take off area – Check clear.

- Observers / Bystanders – Inform take off is imminent.

- Battery power – Monitor.

- Signal Strength – Monitor.

- Motors – On.  Move both control stick down and in, or down and out.

- Takeoff – Engage Auto Takeoff, or move left control stick up.

- Hover check – Complete.  Hover for a period of 20-30 seconds.  Look and listen for any flight abnormalities.

- Home Point – Check, update as required.

MAVIC AIR LANDING

- Landing area – Check clear, announce to observers / bystanders intent to land.

- Return to Home (RTH) – activate if required or desired.  If not used, land aircraft manually.

AFTER LANDING

- Mavic Air – Power down.

- Gimbal security clamp – Install.

- Gimbal cover – Install.

- Transmitter – Power down, store.

- Mavic Air – Remove battery, propellers, and fold arms for storage.  Inspect aircraft for damage, and clean if required.

This Page Intentionally Left Blank

AIRCRAFT STATUS INDICATOR LIGHTS

NORMAL

R – G – Y	Alternate red, green, and yellow flashes.

Turning on, self-diagnostic testing.

Y – G 		Alternate yellow and green flashing.

Warming up.

G – SLOW 	Slow green flashing.

P-mode with GPS

G X 2		Two green flashes.

P-mode with forward and downward vision position.

Y – SLOW	Slow yellow flashing.

No GPS and forward and downward vision systems.

G – FAST	Fast green flashing.

Braking.

AIRCRAFT STATUS INDICATOR LIGHTS

WARNING

Y – FAST	Fast yellow light.

Remote control signal lost.

R – SLOW 	Slow red flashing.

Low battery warning.

R – FAST	Fast red flashing.

Critical low battery warning.

R – FLASH	Red flashing.

IMU Error.

R – SOLID	Solid red.

Critical Error.

R – Y		Fast alternating red and yellow flashing.

Compass calibration required.